# Switch Abstraction Interface

# Change Proposal

| Title | Uniform tunnel API’s |
| --- | --- |
| Authors | Mellanox , metaSwitch |
| Status | First draft |
| Type | Standards Track |
| Created | 08/01/2015 |
| SAI-Version | V0.9.4 |

Contents

List of Changes	i

1	Overview	1

1.1	Uniform tunnel model	1

1.2	IPinIP tunnel	1

1.2.1	IPinIP  encap	1

1.2.2	IPinIP  decap	2

1.3	Vxlan tunnel	2

1.3.1	Vxlan  encap	2

1.3.2	Vxlan  decap	3

1.4	MPLS tunnel	3

1.4.1	Ingres LER (MPLS encap)	3

1.4.2	LSR enacp	4

1.4.3	Egress  LER (MPLS decap)	4

# List of Changes

| Version | Changes | Name | Date |
| --- | --- | --- | --- |
| 0.9.4 | Proposal for uniform tunnel |  | 8/1/15 |

License

© 2014 Microsoft Corporation, Dell Inc., Facebook, Inc, Broadcom Corporation, Intel Corporation, Mellanox Technologies Ltd.

As of September 9, 2014, the following persons or entities have made this Specification available under the Open Web Foundation Final Specification Agreement (OWFa 1.0), which is available at http://www.openwebfoundation.org/legal/the-owf-1-0-agreements/owfa-1-0

Microsoft Corporation, Dell Inc., Facebook, Inc, Intel Corporation, Mellanox Technologies Ltd.

You can review the signed copies of the Open Web Foundation Agreement Version 1.0 for this Specification at http://opencompute.org/licensing/, which may also include additional parties to those listed above.

Your use of this Specification may be subject to other third party rights. THIS SPECIFICATION IS PROVIDED "AS IS." The contributors expressly disclaim any warranties (express, implied, or otherwise), including implied warranties of merchantability, noninfringement, fitness for a particular purpose, or title, related to the Specification. The entire risk as to implementing or otherwise using the Specification is assumed by the Specification implementer and user. IN NO EVENT WILL ANY PARTY BE LIABLE TO ANY OTHER PARTY FOR LOST PROFITS OR ANY FORM OF INDIRECT, SPECIAL, INCIDENTAL, OR CONSEQUENTIAL DAMAGES OF ANY CHARACTER FROM ANY CAUSES OF ACTION OF ANY KIND WITH RESPECT TO THIS SPECIFICATION OR ITS GOVERNING AGREEMENT, WHETHER BASED ON BREACH OF CONTRACT, TORT (INCLUDING NEGLIGENCE), OR OTHERWISE, AND WHETHER OR NOT THE OTHER PARTY HAS BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGE.

THE FOLLOWING IS A LIST OF MERELY REFERENCED TECHNOLOGY: Microprocessor technology, semiconductor manufacturing technology, operating system technology (including without limitation networking operating system technology), emulation technology, graphics technology, video technology, integrated circuit packaging technology and the like, compiler technologies, object oriented technology, optical/RF communications technology including chip I/O and driver technology, bus technology, memory chip technology (including, without limitation, NAND memory, NOR memory, resistive RAM (RRAM), seek scan probe (SSP) memory, nonvolatile memory (including without limitation, memory based on chalcogenide materials, phase change memory (PCM), one or more stacked layers of memory cells, embedded PCM memories, non-volatile cache memory, solid state drives, SRAM, embedded DRAM, ferro-electric memory, and polymer memory)) and/or health-related and medical technology. IMPLEMENTATION OF THESE TECHNOLOGIES MAY BE SUBJECT TO THEIR OWN LEGAL TERMS.

# Overview

# Uniform tunnel object

# Uniform tunnel encap

## IPinIP tunnel

## IPinIP  encap

### IPinIP  decap

## Vxlan tunnel

### Vxlan  encap

### Vxlan  decap

## MPLS tunnel

### MPLS encap

### MPLS  decap

# Specification

New object needed:

Next hop:

Tunnel decap,

Tunnel encap,

MPLS next-hop(NHLFE)

Generic Tunnel object

## Next hop

/**

*  @brief Next hop type

*/

typedef enum _sai_next_hop_type_t

{

SAI_NEXT_HOP_IP,

/*MPLS(NHLFE) next hop */

SAI_NEXT_HOP_MPLS,

/*tunnel next hop */

SAI_NEXT_HOP_TUNNEL_DECAP,

SAI_NEXT_HOP_TUNNEL_ENCAP,

} sai_next_hop_type_t;

typedef enum _sai_next_hop_attr_t

{

/** READ-ONLY */

/** READ-WRITE */

/** Next hop entry type [sai_next_hop_type_t] (MANDATORY_ON_CREATE|CREATE_ONLY) */

SAI_NEXT_HOP_ATTR_TYPE,

/** Next hop entry ipv4 address [sai_ip_address_t]

* (MANDATORY_ON_CREATE when SAI_NEXT_HOP_ATTR_TYPE = SAI_NEXT_HOP_IP)

* (CREATE_ONLY) */

SAI_NEXT_HOP_ATTR_IP,

/** Next hop entry router interface id [sai_object_id_t] (MANDATORY_ON_CREATE|CREATE_ONLY) */

SAI_NEXT_HOP_ATTR_ROUTER_INTERFACE_ID,

/* -- */

/** Next hop entry tunnel-dst [sai_object_id_t]

* (MANDATORY_ON_CREATE when SAI_NEXT_HOP_ATTR_TYPE=SAI_NEXT_HOP_TUNNEL_ENCAP)

* (CREATE_ONLY) */

SAI_NEXT_HOP_ATTR_TUNNEL_DST,

/** Next hop entry tunnel-id [sai_object_id_t]

* (MANDATORY_ON_CREATE when SAI_NEXT_HOP_ATTR_TYPE = SAI_NEXT_HOP_TUNNEL_DECAP | SAI_NEXT_HOP_TUNNEL_ENCAP)

* (CREATE_ONLY) */

SAI_NEXT_HOP_ATTR_TUNNEL_ID,

/** Custom range base value */

SAI_NEXT_HOP_ATTR_CUSTOM_RANGE_BASE  = 0x10000000

} sai_next_hop_attr_t;

## } sai_next_hop_api_t;

## Tunnel object

typedef enum _sai_tunnel_type_t

{

SAI_TUNNEL_IPINIP,

SAI_TUNNEL_IPINIP_GRE,

SAI_TUNNEL_IPINIP_GRE_AND_KEY

SAI_TUNNEL_VXLAN,

SAI_TUNNEL_MPLS,

…

} sai_tunnel_type_t;

typedef enum _sai_tunnel_ttl_mode_t

{

SAI_TUNNEL_TTL_COPY_FROM_INNER,

SAI_TUNNEL_TTL_USER_DEFINE

} sai_tunnel_ttl_mode_t

typedef enum _sai_tunnel_dscp_mode_t

{

SAI_TUNNEL_DSCP_COPY_FROM_INNER,

SAI_TUNNEL_DSCP_USER_DEFINE

} sai_tunnel_dscp_mode_t

typedef enum _sai_tunnel_encap_t

{

/** READ-WRITE */

/** tunnel ip verssion ipv4/ipv6  (MANDATORY_ON_CREATE when SAI_TUNNEL_ATTR_TYPE=SAI_TUNNEL_IPINIP,SAI_TUNNEL_IPINIP_GRE,SAI_TUNNEL_IPINIP_GRE_AND_KEY)

(CREATE_ONLY) */

SAI_TUNNEL_IP_VER,

/**  tunnel src ip (MANDATORY_ON_CREATE when  SAI_TUNNEL_ATTR_TYPE=SAI_TUNNEL_IPINIP,SAI_TUNNEL_IPINIP_GRE,SAI_TUNNEL_IPINIP_GRE_AND_KEY)

(CREATE_ONLY) */

SAI_TUNNEL_SRC_IP,

/**  tunnel TTL mode (copy from inner or user define  [sai_tunnel_ttl_mode_t] MANDATORY_ON_CREATE when  SAI_TUNNEL_ATTR_TYPE=SAI_TUNNEL_IPINIP,SAI_TUNNEL_IPINIP_GRE,SAI_TUNNEL_IPINIP_GRE_AND_KEY)

(CREATE_ONLY) */

SAI_TUNNEL_TTL_MODE,

/**  tunnel TTL value  MANDATORY_ON_CREATE when  SAI_TUNNEL_TTL_MODE = SAI_TUNNEL_TTL_USER_DEFINE)

SAI_TUNNEL_TTL_VAL,

/**  tunnel dscp  mode (pipe or uniform model )  [sai_tunnel_dscp_mode_t] MANDATORY_ON_CREATE when  SAI_TUNNEL_ATTR_TYPE=SAI_TUNNEL_IPINIP,SAI_TUNNEL_IPINIP_GRE,SAI_TUNNEL_IPINIP_GRE_AND_KEY)

(CREATE_ONLY) */

SAI_TUNNEL_DSCP_MODE,

/**  tunnel DSCP value  MANDATORY_ON_CREATE when  SAI_TUNNEL_DSCP_MODE = SAI_TUNNEL_DSCP_USER_DEFINE)

SAI_TUNNEL_DSCP_VAL,

/**  tunnel ECN mapping */

SAI_TUNNEL_ENCAP_ECN_MAPPING,(TBD)

/** tunnel GEP key (MANDATORY_ON_CREATE when SAI_TUNNEL_ATTR_TYPE=SAI_TUNNEL_IPINIP_GRE_AND_KEY)

(CREATE_ONLY) */

*/

SAI_TUNNEL_GRE_KEY,

} sai_tunnel_encap_t;

typedef enum _sai_tunnel_decap_t

{

/** READ-WRITE */

/**  enable decap src ip validation check  MANDATORY_ON_CREATE when     SAI_TUNNEL_ATTR_TYPE=SAI_TUNNEL_IPINIP,SAI_TUNNEL_IPINIP_GRE,SAI_TUNNEL_IPINIP_GRE_AND_KEY)

*/

SAI_TUNNEL_DECAP_SIP_CHECK,

/**  expected tunnel src ip (MANDATORY_ON_CREATE when  SAI_TUNNEL_DECAP_SIP_CHECK is enabled)

(CREATE_ONLY) */

SAI_TUNNEL_EXPECTED_SRC_IP,

/**  tunnel decap ECN mapping */

SAI_TUNNEL_DECAP_ECN_TABLE,(TBD)

} sai_tunnel_decap_t;

typedef enum _sai_tunnel_id_t

{

/** READ-WRITE */

/** tunnel type [sai_tunnel_type_t] (MANDATORY_ON_CREATE|CREATE_ONLY) */

SAI_TUNNEL_ATTR_TYPE;

/** tunnel underlay interface [sai_object_id_t] */

SAI_TUNNEL_UNDERLAY_INTERFACE;

/** tunnel overlay interafce [sai_object_id_t] */

SAI_TUNNEL_OVERLAY_INTERFACE;

/** tunnel encap attribute   [sai_tunnel_encap_t] (MANDATORY_ON_CREATE|CREATE_ONLY) */

SAI_TUNNEL_ENCAP_ATTR;

/** tunnel dencap attribute    [sai_tunnel_decap_t] (MANDATORY_ON_CREATE|CREATE_ONLY) */

SAI_TUNNEL_DECAP_ATTR;

} sai_tunnel_id_t;

/**

*  @brief Attribute id for next hop

*/

typedef enum _sai_next_hop_attr_t

{

/** READ-ONLY */

/** READ-WRITE */

/** Next hop entry type [sai_next_hop_type_t] (MANDATORY_ON_CREATE|CREATE_ONLY) */

SAI_NEXT_HOP_ATTR_TYPE,

/** Next hop entry ipv4 address [sai_ip_address_t]

* (MANDATORY_ON_CREATE when SAI_NEXT_HOP_ATTR_TYPE = SAI_NEXT_HOP_IP)

* (CREATE_ONLY) */

SAI_NEXT_HOP_ATTR_IP,

/** Next hop entry router interface id [sai_object_id_t] (MANDATORY_ON_CREATE|CREATE_ONLY) */

SAI_NEXT_HOP_ATTR_ROUTER_INTERFACE_ID,

/* -- */

/** Custom range base value */

SAI_NEXT_HOP_ATTR_CUSTOM_RANGE_BASE  = 0x10000000

} sai_next_hop_attr_t;

/**

* Routine Description:

*    @brief Create next hop

*

* Arguments:

*    @param[out] tunnel_id - tunnel id

*    @param[in] attr_count - number of attributes

*    @param[in] attr_list - array of attributes

*

* Return Values:

*    @return SAI_STATUS_SUCCESS on success

*            Failure status code on error

*

* Note: IP address expected in Network Byte Order.

*/

typedef sai_status_t (*sai_create_tunnel_fn)(

_Out_ sai_object_id_t* tunnel_id,

_In_ uint32_t attr_count,

_In_ const sai_attribute_t *attr_list

);

/**

* Routine Description:

*    @brief Remove next hop

*

* Arguments:

*    @param[in] tunnel_id – tunnel id

*

* Return Values:

*    @return SAI_STATUS_SUCCESS on success

*            Failure status code on error

*/

typedef sai_status_t (*sai_remove_tunnel_fn)(

_In_ sai_object_id_t tunnel_id

);

/**

* Routine Description:

*    @brief Set Next Hop attribute

*

* Arguments:

*    @param[in] tunnel_id - tunnel id

*    @param[in] attr - attribute

*

* Return Values:

*    @return SAI_STATUS_SUCCESS on success

*            Failure status code on error

*/

typedef sai_status_t (*sai_set_tunnel_attribute_fn)(

_In_ sai_object_id_t tunnel_id,

_In_ const sai_attribute_t *attr

);

/**

* Routine Description:

*    @brief Get tunnel attribute

*

* Arguments:

*    @param[in] tunnel _id - tunnel id

*    @param[in] attr_count - number of attributes

*    @param[inout] attr_list - array of attributes

*

* Return Values:

*    @return SAI_STATUS_SUCCESS on success

*            Failure status code on error

*/

typedef sai_status_t (*sai_get_tunnel_attribute_fn)(

_In_ sai_object_id_t tunnel_id,

_In_ uint32_t attr_count,

_Inout_ sai_attribute_t *attr_list

);

/**

*  @brief Next Hop methods table retrieved with sai_api_query()

*/

typedef struct _sai_tunnel_api_t

{

sai_create_tunnel_fn        create_tunnel;

sai_remove_tunnel_fn        remove_tunnel;

sai_set_tunnel_attribute_fn set_tunnel_attribute;

sai_get_tunnel_attribute_fn get_tunnel_attribute;

} sai_tunnel_api_t;

## Examples

## VXlan

## TBD

## IPinIP

## TBD